BudgetBrailler Instructions

Button Functions from Left to Right

Row 1: Dot 3 Button, Dot 2 Button, Dot 1 Button, Dot 4 Button, Dot 5 Button, Dot 6 Button.

Row 2: Line Up Button, Line Down Button, Home Button, Print Button, No Function, Backspace Button.

Row 3: Space Button, Space Button

Turning on the Brailler

- Plug in keyboard at the back left of the brailler, ensuring pins are in the correct orientation. This will depend on what kind of connectors you used.

- Plug in the power supply to the round barrel jack at the back left of the brailler.

- Listen for a beep from the buzzer and wait for it to stop. This indicates that the brailler is ready.

Feeding Paper

- Push paper in from the from front of the brailler on top of and parallel to the flat surface with the embossing channel on it.

- Once the paper contacts the rollers, press the Line Down Button. Repeat pressing the Line Down Button until the location on the paper where you want to start printing is above the embossing channel. Press the Line Up Button to move the paper back out the front of the brailler if you feed the paper too far or need to restart because the paper is crooked.

Navigating on the Page

- Press the Home Button if necessary to return the carriage to the beginning of the line. This will also delete all the characters you currently have typed but not printed.

- Press the Line Up Button to go up a line on the page, feeding the paper out of the back of the brailler.

- Press the Line Down Button to go down a line on the page, feeding the paper out of the front of the brailler.

Typing Braille

Note: Characters cannot be typed while the brailler is taking another action, such printing, homing, or moving the paper. If one of these actions is started, wait for the buzzer in the brailler to stop to indicated that the brailler is ready again.

- At the same time, press all the Dot Buttons which correspond to the character you wish to type then release them all. Press one or both Space Buttons and none of the Dot Buttons for an empty cell.

- Press the Backspace Button delete the last character typed. This will not allow you to go passed the last printed character if the Print Buffer Button was used or the beginning of the line if the Print Buffer Button was not used.

- Press the Print Buffer Button to print all the characters you have typed or type a complete line of 28 characters and the brailler will automatically print the whole line.